INVITATION TO BID

FOR THE

FY 2019 MOWING AND SHOULDER MAINTENANCE ON VARIOUS ROADS THROUGHOUT THE COUNTY

SOLICITATION NUMBER: 19-001

LET DATE: March 5, 2019

# Douglas County Board of Commissioners

# Purchasing Department

# 8700 Hospital Drive

Douglasville, GA 30134

TABLE OF CONTENTS

| SECTION ONE |  |  |
| --- | --- | --- |
| Advertisement for Bidders |  | 3-4 |
|  | Notice to Contractors……………………………………………….. | 5-6 |
|  | Bid Proposal………………………………………………………… | 7-9 |
|  | Bid Bonds…………………………………………………………… | 10-11 |
|  | Non-Collusion Affidavit………………………………………….... | 12-13 |
|  | *Bid Schedule Items……………………………………………….. | 14-16 |
| SECTION TWO-GENERAL CONDITIONS |  |  |
|  | Definitions and Terms……………………………………………..... | 17-18 |
| SECTION THREE - AGREEMENT |  |  |
|  | *Agreement…………………………………………………………. | 19-20 |
|  | *100% Performance Bond………………………………………….. | 21-22 |
|  | *100% Labor and Material Payment Bond………………………..... | 23-25 |
| ATTACHMENT A – Tractor Mowing and Trimming Specifications |  |  |
|  | General……………………………………………………………… | 26 |
|  | Scope of Work………………………………………………………. | 26 |
|  | General Contract Information……………………………………….. | 26-32 |
|  | Mowing Requirements………………………………………………. | 33-38 |
| ATTACHMENT B – DESIGNATED STREETS |  |  |
|  | Designated Streets (Excel Spreadsheet)……………………………... | 39-42 |
| ATTACHMENT C |  |  |
|  | Evaluation Criteria for Tractor Mowing and Trimming Contract…... | 43 |

Prime Contractor’s Work Authorization Certification

Subcontractor’s Work Authorization Certification

*Requires Signature

ADVERTISEMENT FOR BIDDERS

Sealed proposals will be received by the Douglas County Purchasing Department located at 8700 Hospital Drive, Douglasville, Georgia 30134 until Friday, March 29, 2019 at 2:00 PM:

For the project known as:

FY2019 Mowing and Shoulder Maintenance

on Various Roads Throughout the County

Solicitation Number 19-001

Proposals will be opened and read aloud at the 3rd Floor Conference Room, 8700 Hospital Drive, Douglasville, Georgia 30134 on Friday, March 29, 2019, shortly after 2:00 PM EST. Proposals received after the designated time will not be considered. The OWNER of the project is the Douglas County Board of Commissioners.

The approximate extent and character of the work is generally described as follows:

Three (3) cycles of right-of-way mowing on approximately 260 miles of County roadways with the option to add or remove cycles as final budget dictates, which includes furnishing all materials, labor, and equipment for mowing complete including brush/slope trimming.

Also pricing add alternate roads for State Routes in Douglas County

Bidders shall inform themselves concerning Georgia Laws and comply with same.

Bidding Documents are available for review and purchase by prospective Bidders at the Douglas County Purchasing Office, 8700 Hospital Drive, Douglasville, Georgia 30134.

Bidding Documents are open to the public at the Douglas County Purchasing Office, 8700 Hospital Drive, Douglasville, Georgia 30134. A complete set of documents may be downloaded from the County’s website, www.celebratedouglascounty.com, under the Purchasing Department.

The OWNER is not obligated to consider a Bidder's proposal, if the Bidder does not meet the following criteria:

- Valid proof of Insurance, equal to or exceeding the Owner's requirements.

- List of three references where the Bidder has performed similar work.  Do not include the Owner as one of the references.

- The contractor must be able to bond 100% of the cost of the submitted bid, through a surety company with the qualifications listed below.

The time allowed for completion will be explained in the Specifications.

Each Bid must be accompanied by a Bid Bond with good and sufficient surety or sureties approved by the owner for faithful acceptance of the contract, payable to, in favor of, and for the protection of the OWNER in an amount equivalent to five percent (5%) of the total amount payable by the terms of the contract or, in lieu thereof, in the form of a certified check, cashier's check, or cash in equal amount.

The Successful Bidder will be required to furnish performance and payment bonds with the executed Agreement meeting the requirements of the Contract Documents and executed on the forms attached to the Agreement. The terms and time for payment are set forth in the Agreement.

Surety and insurance companies must have an AM Best rating of A-6 or greater, be listed in the Federal Registry of Companies holding Certificates of Authority and Acceptable Sureties on Federal Bonds, and be licensed by the Georgia Insurance Department and the Georgia Secretary of State to do business in the State of Georgia.

Bidder will provide a list of sub-contractors that will be used in conjunction with this project. Bidder must perform 70% or more of the project, without the use of sub-contractors.

All Bids will remain subject to acceptance for 60 days after the day of the Bid opening, but OWNER may, in its sole discretion, release any Bid and return the Bid security prior to that date.

Each bid must be submitted in a SEALED ENVELOPE, addressed to the OWNER. Each sealed envelope containing a Bid must be plainly marked on the outside as, "FY2019   Mowing and Shoulder Maintenance on Various Roads Throughout the County, Solicitation No: 19-001". If bid is forwarded by mail, the sealed envelope containing the Bid must be enclosed in a separate mailing envelope to the attention of the OWNER at the address previously given.

The Owner reserves the right to reject any or all Bids, to waive formalities and re-advertise. The owner may award the contract as a whole or in parts to one or more successful bidders.

Project Manager:

LaVon King

Maintenance Division Manager

Douglas County Department of Transportation

8249 Chicago Avenue

Douglasville, GA 30134

678-626-0158

SECTION ONE

NOTICE TO CONTRACTORS

Sealed proposals will be received by the Board of Commissioners of Douglas County, Georgia, at the Douglas County Purchasing Office, located at 8700 Hospital Drive, Douglasville, Georgia 30134 unti1 2:00 PM EST, March 29, 2019. NO BIDS WILL BE ACCEPTED AFTER THE 2:00 PM EST DEADLINE, March 29, 2019.   No submitted bid may be withdrawn after the scheduled bid opening for a period of sixty (60) days.

The bids will be publicly opened and read aloud shortly after 2:00 PM, March 29, 2019 in the Douglas County Purchasing Office (3rd Floor) Conference Room at 8700 Hospital Drive, Douglasville, Georgia 30134.

The work to be done consists of the furnishing of all materials, labor and equipment for:

FY2019 Mowing and Shoulder Maintenance

on Various Roads Throughout the County

Solicitation Number 19-001

The Bidder is required to submit all pages of the Bid Proposal.

Bidding Documents are open to the public at the Douglas County Purchasing Office, 8700 Hospital Drive, Douglasville, Georgia 30134. A complete set of documents may be downloaded from the County’s website, www.celebratedouglascounty.com, under the Purchasing Department. Bids should be delivered to:

Douglas County Purchasing Office

8700 Hospital Drive

Douglasville, Georgia 30134

Telephone: (770) 920-7579

All work performed for this project will be in accordance with the Georgia Department of Transportation Standard Specifications, Construction of Roads and Bridges, Current Edition, and any modifications identified in the bid documents.

The Bidder shall not subcontract, transfer, assign, or otherwise dispose of the contract or any portion thereof, without the written consent of Douglas County.

The successful bidder shall secure and pay for necessary approvals, permits, assessments, and charges required for the construction and installation of this project as required by local, state, and federal regulations.

The Douglas County Board of Commissioners reserves the right to reject all bids and to waive formalities. The owner may award the contract as a whole or in parts to one or more successful bidders.

Any claims for cost incurred by any Bidder in preparation of any part of or total package for this project will not be considered for reimbursement by Douglas County.

BID PROPOSAL

Page 1 of 3

Proposal of _____________________________________ (hereinafter   called   "Bidder") a contractor organized and existing under the laws of the State of ________________________, *an individual,      a       corporation,       or         partnership        doing         business       as ___________________________________________________________________________.

TO:	Douglas County

(Hereinafter called “County")

Gentlemen:

The Bidder in compliance with your Notice to Contractors and all Bidding Documents for the construction of:

FY 2019 Mowing and Shoulder Maintenance

on Various Roads Throughout the County

Solicitation Number 19-001

having examined the plans and specifications with related documents and the site of the proposed work, and being familiar with all of the conditions surrounding the construction of the proposed project, including the availability of materials and supplies to construct the project in accordance with the contract documents, within the time set forth herein, and at the prices stated below, proposes to enter into a Contract with Douglas County to provide the necessary machinery, tools, apparatus, all materials and labor, and other means of construction necessary to complete the Work. The undersigned proposes to furnish and construct the items listed in the attached Schedule of Items for the unit prices stated.

Bidder agrees that the cost of any work performed, materials furnished, services provided or expenses incurred, which are not specifically delineated in the Contract Documents, but which are incidental to the scope, intent, and completion of the Contract, shall be deemed to have been included in the prices bid for the various items scheduled.

Bidder further proposes and agrees hereby to promptly commence the Work with adequate force and equipment within ten (10) calendar days from receipt of Notice to Proceed, or as may be specified by Special Provision, and to complete the Work as stated within these documents. The original contract period will be for one fiscal year;

BID PROPOSAL

Page 2 of 3

pending budget approval per year two additional one-year extensions will be granted.  This is assuming the performance of the Contractor is satisfactory to the Department of Transportation Representative.

The Bidder acknowledges receipt of the following addenda:

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

The undersigned further agrees that in case of failure on his part to execute said contract and bonds, or provide satisfactory proof of carriage of the insurance required, within ten (10) calendar days after notification of award thereof, the Bid Bond or certified check accompanying his bid and the money payable thereon shall be forfeited to the County as liquidated damages; otherwise, the check or Bond accompanying this proposal shall be returned to the Bidder.

The Bidder declares that he understands that the quantities shown on the proposal are subject to adjustment by either increase or decrease, and that should the quantities of any of the items of work be increased, the undersigned proposes to do the additional work at the unit prices stated herein; and should the quantities be decreased, he also understands that payment will be made on actual quantities at the unit price bid and will make no claim for anticipated profits for any decrease in the quantities, and that actual quantities will be determined upon completion of work, at which time adjustment will be made to the Contract amount by direct increase or decrease.

Attached hereto is a bid bond or certified check on the (Bank)________________________

in the amount of ________________________________________, (Five percent of Total Amount Bid).

BID PROPOSAL

Page 3 of 3

The full name and residence of persons or parties interested in the foregoing bid as principals are named as follows:

| ____________________________________ | ____________________________________ |
| --- | --- |
| ____________________________________ | ____________________________________ |
| ______________________________________ | ____________________________________ |
| ____________________________________ | ____________________________________ |

Douglas County, Georgia

Signed, sealed, and dated this _______day of ________________________, 2019.

Bidder: _________________________________                                                                                                                                           (Company Name)

By:_____________________________________

Title:___________________________________

Mailing Address:________________________________________

________________________________________

County __________________________________

Phone __________________________________

BID BOND

Page 1 of 2

(Five Percent (5%) of Bid)

KNOW ALL MEN BY THESE PRESENTS, that we, the undersigned ________________________________________________________,as Principal, and ____________________________________________, as Surety, are hereby held and firmly bound unto    DOUGLAS COUNTY,   GEORGIA, as     County   in    the penal   sum    of  _______________________________________ Dollars ($ _____________) for the payment of which, well and truly to be made, we hereby jointly and severally bind ourselves, our heirs, executors, administrators, successors, and assigns.

Signed this _______day of _________________, 2019.

The condition of the above obligation is such that whereas the Principal has submitted to DOUGLAS COUNTY, GEORGIA a certain bid attached hereto and hereby made a part hereof to enter into a contract in writing for the construction of:

FY2019 Mowing and Shoulder Maintenance

on Various Roads Throughout the County

Solicitation Number 19-001

NOW, THEREFORE,

- If said bid shall be rejected or in the alternate,

- If said bid shall be accepted and the Principal shall execute and deliver a Contract in the Form of Contract attached hereto (properly complete in accordance with said bid) and shall furnish a bond for his faithful performance of said Contract and for the payment of all persons performing labor or furnishing material in connection therewith, and shall in all other respects perform the agreement created by the acceptance of said bid, then this obligation shall be void, otherwise the same shall remain in force and effect; it being expressly understood and agreed that the liability of the surety for any and all claims hereunder shall, in no event, exceed the penal amount of this obligation as herein stated.

The Surety, for value received, hereby stipulates and agrees that the obligations of said Surety and its bond shall be in no way impaired or affected by any extension of the time within which the County may accept such bids, and said Surety does hereby waive notice of any such extension.

BID BOND

Page 2 of 2

IN WITNESS WHEREOF, the Principal and the Surety have hereunto set their hands and seals, and such of them as are corporations have caused their corporate seals to be hereto affixed and these presents to be signed by their proper officers, the day and year first set forth above.

_____________________________________(SEAL)

Principal

By:

_____________________________________(SEAL)

Surety

By:

Georgia  Representative

NONCOLLUSION AFFIDAVIT OF PRIME BIDDER

State of ___________________)

) ss.

County of _________________)

,being first duly sworn, deposes and says that:

- He is __________________ (Owner, Partner, Officer, Representative, or Agent) of the Bidder that has submitted the attached Bid;

- He is fully informed respecting the preparation and contents of the attached Bid and of all pertinent circumstances respecting such Bid;

- Such Bid is genuine and is not a collusive or sham Bid;

- Neither the said Bidder nor any of its officers, partners, owners, agents, representatives, employees, or parties in interest, including this affiant, has in any way colluded, conspired, connived, or agreed,  directly or indirectly, with any other Bidder, firm or person to submit a collusive or sham Bid in connection with the Contract for which the attached Bid has been submitted to or refrain from bidding in connection with such Contract, or has in any collusion or communication or conference with any other Bidder, firm or person to fix the price or prices in the attached Bid or of any other Bidder, or to fix any overhead, profit or cost element of the Bid price or the Bid price of any other Bidder, or to secure through any collusion, conspiracy, connivance or unlawful agreement any advantage against Douglas County or any person interested in the proposed Contract; and,

- The price or prices quoted in the attached Bid are fair and proper and are not tainted by any collusion, conspiracy, connivance, or unlawful agreement on the part of the Bidder or any of its agents, representatives, owners, employees, or parties in interest, including this affiant.

(Signatures on next page)

Signed    _______________________________

Name    ________________________________

Title    _________________________________

Subscribed and Sworn to before me this __ day of _______________________, 2019.

(SEAL)

____________________________________________

Title

My Commission expires

Date _________________.

Douglas County Board of Commissioners

Department of Transportation

Base Bid Schedule of Items

____________________________________________________________________

Solicitation No.: 19-001		Vendor: ___________________________________

Signature: ________________________________

Contract Scope:		FY2019 MOWING AND SHOULDER MAINTENANCE

ON VARIOUS ROADS THROUGHOUT THE COUNTY

Total For County Roads & Hours

| Line No. | Item Description | Unit | Approx. Quantity | Unit Price | Amount |  |
| --- | --- | --- | --- | --- | --- | --- |
| 01 COUNTY ROADS (SEE Schedule “C”) |  |  |  |  |  |  |
| 10 | Cycle 1 – Mowing Complete | CLM | 260 |  |  |  |
| 20 | Cycle 2 - Mowing Complete | CLM | 260 |  |  |  |
| 30 | Cycle 3 – Mowing Complete | CLM | 260 |  |  |  |
|  | Total Cycles 1-3 | CLM | 780 | $ |  |  |
| 40 | Optional Vertical Mowing | HR | 181 |  |  |  |
|  | Total Base Bid |  |  | $ |  |  |

Mowing Complete includes mowing limits as defined in the contract, etc.

Traffic Control shall be included in the priced bid for each work item.

TOTAL BASE BID            $ ___________________

"END OF SECTION"

SECTION TWO

SCHEDULE “C”

Add Alternate for State Routes in Douglas County

| Road | Start | End | CLM |
| --- | --- | --- | --- |
| Hwy 78 | City of Villa Rica | Cobb County Line | 16.5 |
| Hwy 5 | Hwy 78 | Carroll County Line | 14.5 |
| Hwy 166 | Hwy 92 | Carroll County Line | 17.5 |
| Hwy 92 | Hwy 78 | Fulton County Line | 13.5 |
| Hwy 6 ( Thornton Rd ) | Cobb County Line | Fulton County Line | 6.5 |
| Interstate I -20 | Carroll County Line | Cobb County Line | 19.0 |
|  |  | Total | 87.5 |

Bid Schedule of Items – Add Alternate

____________________________________________________________________

Solicitation No.: 19-001		Vendor: ___________________________________

Signature: ________________________________

Contract Scope:	          FY2019 MOWING AND SHOULDER MAINTENANCE

ON VARIOUS ROADS THROUGHOUT THE COUNTY

Add Alternate For State Routes In Douglas County

| Line No. | Item Description | Unit | Approx. Quantity | Unit Price | Amount |
| --- | --- | --- | --- | --- | --- |
| STATE ROADS (See Schedule “C”) |  |  |  |  |  |
| 1 | Cycle 1 – Mowing Complete | CLM | 87.5 |  |  |
| 2 | Cycle 2 - Mowing Complete | CLM | 87.5 |  |  |
| 3 | Cycle 3 – Mowing Complete | CLM | 87.5 |  |  |
| 4 | Cycle 4 – Mowing Complete | CLM | 87.5 |  |  |
|  | Total Cycles 1-4 | CLM | 350.0 | $ |  |

Mowing Complete includes mowing limits as defined in the contract, including highway medians where they exist, etc.

Traffic Control shall be included in the priced bid for each work item.

TOTAL FOR STATE ROUTES ADD ALTERNATE            $ ___________________

"END OF SECTION"

SECTION TWO

GENERAL CONDITIONS

Unless otherwise directed, all work performed under this contract shall be in accordance with the Georgia Department of Transportation Standard Specifications, Construction of Roads and Bridges, Current Edition, and any current Supplemental Specifications modifying them, except as noted below and in the Special Provisions.

Modifications to Standards Specifications

SECTION 101 	DEFINITION AND TERMS

Section  101.10          BOARD

Delete as written and substitute the following:

BOARD OF COMMISSIONERS OF DOUGLAS COUNTY, GEORGIA

Section  101.14           COMMISSIONER

Delete as written and substitute the following:

THE COUNTY ADMINISTRATOR OF DOUGLAS COUNTY

Section  101.22          DEPARTMENT

Delete as written and substitute the following:

THE DOUGLAS COUNTY DEPARTMENT OF TRANSPORTATION

Section  101.24           ENGINEER

Delete as written and substitute the following:

THE DOUGLAS COUNTY DIRECTOR OF TRANSPORTATION, ACTING DIRECTLY OR THROUGH HIS DULY AUTHORIZED REPRESENTATIVE.

Section 101.62            STATE HIGHWAY ENGINEER

Delete as written and substitute the following:

THE DOUGLAS COUNTY DIRECTOR OF TRANSPORTATION, ACTING DIRECTLY OR THROUGH HIS DULY AUTHORIZED REPRESENTATIVE.

Section 101.74          SUPPLEMENTAL AGREEMENT

Retain as written and add the following:

"Any Supplemental Agreement that has a dollar value amount that is less than $25,000.00 shall not require the assent of the Surety."

Section 101.81         TREASURER

Delete as written and substitute the following:

THE DOUGLAS COUNTY FINANCE DIRECTOR, ACTING DIRECTLY OR THROUGH HER DULY AUTHORIZED REPRESENTATIVES.

Add:

Section 101.84

COUNTY:       THE DOUGLAS COUNTY BOARD OF COMMISSIONERS

SECTION THREE

AGREEMENT

THIS  AGREEMENT,  made  this          day  of  _______________ , 2019 by  and  between  the  BOARD  OF COMMISSIONERS    OF    DOUGLAS    COUNTY,    hereinafter    called    "OWNER,"    and  ________________________________, a contractor doing business as a corporation of the City of __________________, County of ___________, and State of _____________________, hereinafter called "Contractor."

WITNESSETH: that for and in consideration of the payments and agreements hereinafter mentioned, to be made and performed by the OWNER, the Contractor hereby agrees to commence and complete the construction described as follows:

FY2019 Mowing and Shoulder Maintenance

On Various Roads Throughout the County

Solicitation Number 19-001

_________________________________________________________________________ _______________________________________________________________________

hereinafter  called the "Project," for the sum of $____________________________                                    and all extra work in connection therewith, under the terms as stated in the General Conditions, special Provisions and Detailed Specifications of the Contract, and at his/her/its/their own proper cost and expense to furnish all materials, supplies, machinery, equipment, tools, superintendence, labor, insurance, and other accessories and services necessary to complete the said project in accordance with the conditions and prices stated in the proposal, the General Conditions, Special Provisions and Detailed Specifications of the Contract, the plans, which include all explanatory matter thereof, as prepared by OWNER, the specifications  and contract documents, all of which are made a part hereof and collectively constitute the Contract.

The Contractor further proposes and agrees hereby to promptly commence the Work with adequate force and equipment within ten (10) calendar days from receipt of each Notice to Proceed, or as may be specified by Special Provision, and to complete the Work within the stipulated time frame of each  Notice to Proceed, based on consecutive calendar days from the Notice to Proceed date.

The OWNER agrees to pay the Contractor in current funds for the performance of the Contract subject to additions and deductions as provided in the General Conditions of the

AGREEMENT (CONTINUED)

Contract, and to make payments on account thereof as provided in the Specifications, as modified in the General Conditions and Special Provisions.

IN WITNESS WHEREOF, the parties to those presents have executed this Contract in three (3) counterparts, each of which shall be deemed an original.

Executed this ________  day of   ______________________ 2019.

BOARD OF COMMISSIONERS OF DOUGLAS COUNTY, GEORGIA

ATTEST:

By: _____________________________          ________________________________

Dr. Romona Jackson-Jones, Chairman            Lisa Watson, County Clerk

____________________________

Witness

ATTEST:

____________________________________		     _____________________________

Contractor                                                                           Secretary

By: _____________________________________                     __________________________________

Witness

Title: __________________________

Approved as to Form by:                                                                                   (Seal)

Give proper title of each person executing affidavit.  Attach seal as required.

100% PERFORMANCE BOND

KNOW ALL MEN BY THESE PRESENTS:  THAT ________________________________ as Principal (hereinafter called Contractor), and ________________________________________

________________________________a  Corporation organized and existing under the laws of the State of    ___________________   As Surety (hereinafter called Surety), are held and firmly bound unto DOUGLAS  COUNTY BOARD OF COMMISSIONERS   as obligee, hereinafter called OWNER, in the amount of ___________________________________($____________.00)  for the payment whereof Contractor and Surety bind themselves, their heirs, executors, administrators, successors and assigns, jointly and severally, firmly by these presents.

WHEREAS, Contractor has by written agreement dated ________________________,                                           2019, entered into a contract with OWNER for:

FY2019 Mowing and Shoulder Maintenance

on Various Roads Throughout the County

Solicitation Number 19-001

in  accordance  with  drawings  and  specifications  prepared  by  Douglas  County,  which  Contract  is  by reference made a part hereof, and is hereinafter referred to as the Contract.

NOW, THEREFORE, the condition of this obligation is such that, if Contractor shall promptly and faithfully perform said Contract, then this obligation shall be null and void, otherwise it will remain in full force and effect.

The Surety hereby waives notice of any alteration or extension of time made by the OWNER.

Whenever Contractor shall be, and declared by OWNER to be in default under the Contract, the OWNER having performed OWNER'S obligation there under, the Surety may promptly remedy the default, or shall promptly

- Complete the contract in accordance with its terms and conditions; or,

- Obtain a bid or bids for completing the Contract in accordance with its terms and conditions, and upon determination by Surety of the lowest bidder, or, if the OWNER

PERFORMANCE BOND

Page 2 of 2

elects, upon determination by the OWNER and Surety jointly of  the lowest responsible bidder, arrange for a  Contract between such Bidder and the OWNER, and make available as work progresses (even though there should be a default or a succession of defaults  under the Contract or Contract of Completion arranged under this paragraph)  sufficient funds to pay the cost of completion less the balance of the Contract price; but not exceeding, including other costs and damages for which the Surety may be liable hereunder, the amount set forth in the first paragraph hereof. The term "Balance of the Contract Price," as used in this paragraph shall mean the total amount payable by OWNER to Contractor under the Contract and any amendments hereto, less the amount properly paid by OWNER to Contractor.

No action can be instituted on this bond after one year from the completion of the Contract and the acceptance by the OWNER of the work thereunder.

Signed and sealed this ________ day of ___________________________, 2019  in the presence of:

_______________________________

Principal

_______________________________          	By: _______________________________ (Seal)

Witness

_______________________________________

Surety

________________________________        	By: ________________________________(Seal)

Witness

100% LABOR AND MATERIAL PAYMENT BOND

KNOW ALL MEN BY THESE PRESENTS:

That ___________________________________  as Principal, hereinafter called Principal, and  ____________________________________  of  _____________, a corporation organized and existing under the laws of the State of ______________  as Surety, hereinafter called Surety, are held and firmly bound unto DOUGLAS   COUNTY  BOARD  OF  COMMISSIONERS, as obligee, hereinafter called OWNER, in the amount of  ________________________________________________Dollars ($                                   .00).

WHEREAS, Principal has by written agreement dated   ______________, 2019, entered into a Contract with OWNER for:

FY2019 Mowing and Shoulder Maintenance

on Various Roads Throughout the County

Solicitation Number 19-001

in accordance with drawings and specifications prepared by Douglas County, which contract is by reference made a part hereof, and is hereafter referred to as the Contract.

NOW, THEREFORE, the condition of this obligation is such that if the Principal shall promptly make payment to all claimants as is herein below defined, for all labor and materials used or reasonably required for use in the performance of the Contract, this obligation shall be void; otherwise, it shall remain in full force and effect subject, however, to the following conditions:

- A claimant is defined as one having a direct contact with the principal or with a subcontractor of the Principal for labor, material, or both, used or reasonably required for use in the performance of the Contract, labor and material being construed to include that part of water, gas, power, light, heating oil, gasoline, telephone service, rental of equipment, or repair of equipment directly applicable to the Contract.

- The above-named Principal and Surety hereby jointly and severally agree with the OWNER that every claimant as herein defined, who has

LABOR AND MATERIAL PAYMENT BOND

Page 2 of 3

not been paid in full before the expiration of a period of ninety (90) days after the date on which the last of such claimant's work or labor was done or performed, or materials were furnished by such claimant, may sue on this bond for the use of such claimant, prosecute the suit to final judgment for such sum or sums as may be justly due claimant, and have execution thereon. The OWNER shall not be liable for the payment of any costs or expenses of any such suit.

- No suit or action shall be commenced hereunder by any claimant.

  - Unless claimant, other than one having a direct Contract with the Principal, shall have given written notice to any two of the following: the Principal, the OWNER, or the Surety above-named, within ninety (90) days after such claimant did or performed the last of the work or labor, or furnished the last of the materials for which said claim is made, stating with substantial accuracy the amount claimed and  the name of the party to whom the materials were furnished, or for whom the work or labor was done or performed. Such notice shall be served by mailing the same by registered mail or certified mail, postage prepaid, in an envelope addressed to the Principal, OWNER, or Surety, at any place where an office is regularly maintained for the transaction of business, or served in any manner in which legal process may be served in the state in which the aforesaid project is located, save that such service need not be made by a public officer.

- After one year from the completion of the Contract and the acceptance by OWNER of the work thereunder, it being understood, however, that if any limitation embodied in this bond is prohibited by any law controlling the construction hereof such limitation shall be deemed to be amended so as to be equal to the minimum period of limitation permitted by such law.

- Other than in a state court of competent jurisdiction in and for the county or other political subdivision of the state in which the project, or any part thereof, is situated, or  in the United States District Court for the district in which the project, or any part thereof, is situated and not elsewhere.

LABOR AND MATERIAL PAYMENT BOND

Page 3 of 3

- The amount of this bond shall be reduced by and to the extent of any payment or payments made in good faith hereunder inclusive of the payment by Surety or mechanics' liens which may be filed of record against improvement, whether or not claim for the amount of such lien be presented under and against this bond.

Signed and sealed this _______ day of _______________________, 2019.

_______________________________

Principal

________________________________                   By: _____________________________(Seal)

Witness

______________________________

Surety

_______________________________

Witness

By: ____________________________(Seal)

Representative

This bond is issued simultaneously with Performance Bond in favor of the OWNER.

ATTACHMENT “A”

DEPARTMENT OF TRANSPORTATION

DOUGLAS COUNTY

TRACTOR MOWING AND TRIMMING SPECIFICATIONS

- GENERAL

The Douglas County Department of Transportation (hereafter referred to as the Department) is seeking bids for tractor mowing and trimming, trash pickup, and weed eradication and removal from road medians within the limits of the right of way and easement areas according to the specifications and requirements found within this document.

- SCOPE OF WORK

Work under these Specifications consists of furnishing labor, material, tools, equipment, and incidentals necessary to perform tractor mowing and trimming operations within the mowing limits of County Routes in a safe, effective, and timely manner, as detailed in these Specifications.  Perform and complete mowing operations as specified.

The Department expects that the successful vendor will have sufficient resources to complete the awarded contract.

ATTACHMENT “B” on pages 40 – 43 of this request for bid package provides locations and estimated amount of centerline miles to be tractor mowed and trimmed for each mowing cycle.

- GENERAL CONTRACT INFORMATION

3.01     TERMS AND DEFINITIONS

  - Approval/ Acceptance - Inspected and approved by the Project Manager for compliance with the requirements of the specifications included in this contract.

  - Bridge End Roll- The area at each end of a bridge that slopes down from the

end support to the natural ground contour under the bridge.

  - Centerline Mile (CLM)- Incremental measurement used in this contract for bidding purposes that includes the mowing limits within a one-mile distance on a route. This includes both directions of the mile, shoulders and median.

  - Liquidated Damages- Failure to comply with terms and specifications of the Contract that will result in monetary liquidation of invoices submitted for payment.

  - Mowing Cycle- A mowing cycle consists of mowing complete the areas as designated in these Specifications, one time.  Complete a Mowing Cycle without interruption, within the time frames listed and to the satisfaction of the Project Manager.

  - Mowing Complete- The accepted/approved tractor mowed and trimmed centerline miles within established mowing limits.

  - Mowing Limits- All areas that are mowable with tractor mowing equipment within the right of way that are on a 3:1 or flatter slope, and trimming in front of, behind, under, and around roadside obstacles, and mowing/trimming on road medians, weed eating of raised islands or roads medians and on all slopes greater than or equal to 3: 1 to 5 feet behind the ditch line on a cut slope and 5 feet past the break point on a fill slope.

- Natural Tree Line- The boundary/edge of a community of hardwood and non-hardwood trees, greater than 3 inches in caliper (dbh) diameter at breast height (measured at 4ft, feet above the ground).  The edge is typically a fluctuating line.

- Roadside Obstacles- Items located within the right of way, both natural and man-made which may include but are not limited to the following:  sign posts, delineator posts, light posts, steel beam guardrail and associated posts, headwalls, barrier walls, retaining walls, catch basins, fallen rock, bridge end abutments, mailboxes, fire hydrants, newspaper boxes, established/planted tree/shrub(s), landscaped beds, natural tree line, wildflower areas, bridge end rolls, and fences.

- Slope Break Point- The spot where the non-mowable slope begins.

- Traveled Way- The portion of the roadway for the movement of vehicles, exclusive of the shoulders

- Shadow Vehicle -A vehicle used to buffer mowing that may interfere with traffic.

  - APPLICABLE  DOCUMENTS

    - REQUIRED REFERENCES:

Reference the guidelines as set forth in the following documents, as they will be used by the Department when determining quality of work and will be applicable to this Specification/Contract:

      - Manual of Uniform Traffic Control Devices (MUTCD), current edition

3.03	AWARD

- Award will be made to the lowest, responsible, bidder meeting the specifications set out herein.

    - Unbalance Bid:

All bids shall be subject to review for prices that are either in excess of our below the reasonable cost as compared to the Engineer's estimate and historical prices kept by the Department. The submittal of an irregular/unbalanced bid may result in the rejection of the Contractor’s bid.

- Administration:

The Douglas County Director of Transportation will administer this contract through his designated representative Project Manager.  The Project Manager’s address is:

Douglas County Department of Transportation

8249 Chicago Avenue

Douglasville, GA  30134

Phone: (678) 626-0158

Fax: (678) 626-0157

- Maintenance Service Period: (Completion Schedule)

Upon receipt of Notice of Award, start work on a date agreed upon between the Contractor and the Project Manager (See Attachment "A"- Section 4.01 for Commencement Dates). Review any required post award submittals. Start work no later than the agreed upon commencement date.

- Measurement:

The area of Mowing Complete will be measured and accepted in Centerline Miles.

- Default/Non-Compliance/Liquidated  Damage

  - When the Department determines that the Contractor has failed to perform required work to the terms of the Contract; the Contractor shall be deemed in Contract Non-Compliance and subject to Contract termination.

- The Department has the authority to terminate the Contract when the Contractor is in Non-Compliance three (3) or more times during the term of the Contract.

- All Non-Compliance of Contract may result in Liquated Damages against the Contractor and begin as described below, and continue daily until the task has been executed to the satisfaction of the Project Manager.

- The following are applicable liquidated damage(s):

| Specification Non-Compliance Item | Section | Liquidated Damages |
| --- | --- | --- |
| Failure to comply with personnel requirement per item 3.04 | Sec. 3.04 | $100.00 per item of non-compliance/per day |
| Failure to provide required Tractor Mowing/Trimming equipment | Sec. 4.03 | $100.00 per item of non-compliance/per day |
| Failure to begin Mowing Cycle on approved, scheduled commencement date | Sec. 4.01 | $500.00 per day |
| Failure to complete Mowing Cycle on intermediate and completion date | Sec. 4.01 | $500.00 per day |
| Failure to comply with Miscellaneous Mowing Complete and Traffic Control Requirements | Sec. 4.04 Sec. 4.05 | $500.00 per item of non-compliance/per day |
| Failure to comply and/or correct Quality Control deficiencies | Sec. 406 | $500.00 per item of non-compliance/per day |

- The Department may suspend work if non-compliance occurs with items listed above, until actions are corrected.

- Performance deficiencies will be reported to the Contractor, by facsimile, within seventy­ two (72) hours of Non-Compliance identification(s) and Liquated Damages occurred. A written copy of the facsimile will be mailed to the Contractor.

- Reply to the Project Manager’s non-compliance facsimile/letter by facsimile within 48 hours of receipt. Any Liquidated Damages assessed will be deducted from the Contractor's invoice for payment. The contractor must dispute any liquidated damages in writing within 48 hours of receipt of notification of assessment.

- Payment:

  - After each cycle is complete, submit to the Project Manager a detailed invoice that reflects the locations and total Centerline Miles mowed. The submitted invoice must utilize milepost designations from point to point of all Centerline Miles completed for the Project Manager’s verification.

  - On a monthly basis, submit to the Project Manager a detailed invoice that reflects the Centerline Miles of roadways that that have been mowed.

  - Payment will be made at the unit price per Centerline Mile and will be measured in accordance with Sec. 3.03.E of these Specifications. This payment shall be full compensation for furnishing all materials, labor, tools, equipment, traffic control, and incidentals necessary to satisfactorily complete the work described.

  - Payment shall be calculated by the Contractor and verified by Project Manager according to the number of Centerline Miles mowed on a complete cycle basis.

  - Payment should be received approximately thirty (30) days after the invoice is received, reviewed, and approved by the Department.

  - In the event of a dispute, provide a detailed measurement of the area in question.

- Insurance:

Submit original certificate of insurance in conformance with prescribed limits within ten (10) calendar days after contract award. Commence no work prior to Department acceptance of insurance coverage. Failure to retain insurance for the term of the contract may be grounds for termination.

- Quantities:

It is understood that this is an Open Agency Contract: meaning, Douglas County Department of Transportation is not required to purchase any maximum or minimum service as estimated or contemplated by the Notice of Award, Contract or attachments. Responsible Department representatives will make every effort by issuing Purchase Orders as releases referencing this contract in order to progress with work activities as scheduled within the contract.

3.04	PERSONNEL

    - Supervisor - Have a competent and experienced Supervisor/Foreman on duty at all times when work is being performed. The Supervisor/Foreman must speak English.

      - This Supervisor/Foreman shall be in a separate support vehicle such as a pickup or service truck, whose duties are strictly supervisory and is not a part of a Tractor Mowing or Trimming crew.

      - The Supervisor/Foreman shall have a functional cellular phone with messaging or call waiting capability, on his person during duty hours. The Supervisor/Foreman's response time to the Project Manager shall be less than 30 minutes.

      - Use a different Supervisor/Foreman for each awarded mowing contract.

    - Crew - Provide staff and staffing levels able to perform the work in accordance with this Specification. The personnel staff will be under the sole responsibility of the Contractor.

      - Assure that all employees are qualified to safely operate mechanized Tractor Mowing and Trimming equipment. Employees will be competent, experienced and skilled in all aspects of Tractor Mowing and /or Trimming work.

- All personnel will have a valid U.S. driver's license.

- The Crew shall be able to communicate with the Supervisor/Foreman at all times while performing work.

    - Dress Code - The minimum dress code for personnel shall be a clean and complete outfit, including pants, shoes, shirt, safety vest and any other necessary safety gear required by local, state or federal regulations.

3.05	SUBMITTALS

Any work performed prior to receiving approved submittals shall be considered unauthorized and will result in non-payment for that work.

    - Work Schedule- Five (5) days prior to each mowing cycle:

      - Submit a plan of how the Mowing Cycle will be completed including number of Tractor Mowing and Trimming crews. Prior to beginning each Mowing Cycle the Project Manager must approve this plan. Multiple mowing and trimming crews will be required.

- Submit a list of active phone numbers including office phone, office fax, and cellular phone numbers.

  - Submit the Supervisor/Foreman's name and cellular phone number.

  - Supply a functional cellular phone for use by the Project Manager for correspondence with the Contractor and/or the Supervisor/Foreman during the term of this Contract.

    - Personnel- Submit a list of all employees who will be working during that Cycle. No work will begin without this submittal.

1. Prior to beginning any work, submit the names of all new employees who are to work during a current Mowing Cycle. This includes subcontractor employees if a subcontractor is approved.

2.  The Department has the right to exclude any person who engages in conduct that is nonprofessional. Replace any person, excluded by the Department, in a prompt manner.

    - Equipment:

Submit a list of all equipment contemplated for use. This equipment shall be subject to inspection and acceptance, at any time, for mechanical worthiness and appropriateness for the work intended by the Project Manager. Department decisions relevant to mechanical worthiness shall be final.

3.06 	SUBCONTRACTORS

    - The Department expects the awarded contractor to be fully able to complete the work as detailed in these specifications. The Department may consider the use of subcontractors to accomplish the work.

- If the Department approved the use of a subcontractor, the General Contractor will be held totally responsible for the actions, quality and timeliness of all work performed by the subcontractor.

- All requests to hire a subcontractor, if approved, must be approved in writing. A minimum of five (5) working days is needed to evaluate a request. The written request from the contractor shall include the following:

  - A justification for the need to hire a subcontractor;

- The name and address of the company proposed as a subcontractor;

- The number of employees proposed;

- The phone number of the subcontractor;

- Proposed locations of work and revised work schedule.

- MOWING REQUIREMENTS

4.01     MOWING CYCLES

  - A Notice to Proceed will be given 10 days prior to beginning work for each cycle of mowing. A completion date will be established with each Notice to Proceed.  There will be a minimum of three (3) mowing cycles on all Routes with an optional forth (4) Cycle.

- Completion Dates:

Group 1			30 Days from NTP

Group 2			30 Days from NTP

Group 3			20 Days from NTP

Group 4 (Optional)		15 Days from NTP

- Mowing Cycle Extensions:  Adjustments may be made for delays caused by weather or other Natural Disasters such as floods, hurricanes or by decision of the Project Manager or his designee.

  - MOWING LIMITS FOR EACH CYCLE

All mowable areas within the right-of-way that are on a 3: 1 flatter slope using tractors with rotary or flail type mowers; and trimming in front of, behind, under and around roadside obstacles, landscaped areas, planted/established trees, and wildflowers plots and trimming on slopes flatter and/or steeper than 3:1. Perform trimming in a manner as to create a smooth transition back to where the Tractor Mowing was performed. Also, on slopes steeper than 3: 1 mow 5 feet behind the back of the ditch in a cut slope and 5 feet past the break point on a fill slope.

4.03        MOWING COMPLETE REQUIREMENTS

    - Tractor Mowing-Perform Tractor mowing within the Mowing Limits to mow vegetation on slopes no steeper that 3:1. This operation will not be allowed on slopes steeper that 3:1, or any other situations that may be a hazard to the operator or the traveling public, including areas that may be too wet or otherwise inaccessible.

    - Equip tractors with two flashing signal lights, two flags on both the tractor and mower, and an 18 inch slow moving emblem sign.

- Acceptable guards are to be on all open portions of tractor Mowers and Trimming equipment to prevent objects from being thrown from under the mower while in operations.

- Properly maintain mower blades, i.e., sharp and straight.

    - Trimming - Trim vegetation to a height of 6 inches within the Mowing Limits that are inaccessible by Tractor Mowing. This includes trimming in front of, behind, under or around, and beyond roadside obstacles, landscape beds, established/planted trees and wildflower areas to provide an attractive appearance. The Project Manager has the right to identify those areas where Trimming shall be required. (See all Attachments F).

      - Mainline Slopes that are Greater that 3: 1 -Each mowing cycle, trim all vegetation to five feet inside the natural tree line. When a natural tree line does not exist trim all vegetation to the right of way.

- Mainline (with slopes less than 3:1) - Trim vegetation to the natural tree line

or to the right of way if no natural three line exists.

- Landscape and wildflower plots- Trim vegetation around landscaped areas, established or planted trees, shrubs and wildflower plots without damaging plant materials. Trim vegetation under planted trees as required by the Project Manager.

- Wet or inaccessible areas- If an area is too wet and/or otherwise inaccessible by tractor mowing equipment, trim vegetation using hand held string, blade type trimmer, small machine, and/or slope mowers.

- Bridge structures- Trim vegetation located at, under, or near bridge structures including bridge end rolls and rip rap areas within the right-of-way.

  - If trees (greater than 3" caliper (dbh) diameter at breast height) exist between adjacent bridges, trim all vegetation under and around trees.

  - Trim vegetation on bridge ends and bridge end rolls at overpasses and underpasses to the right of way fence and/or natural tree line. If trees exist within 15 feet around/adjacent to the bridge, then trim vegetation under and around the trees, or as directed by the Project Manager. Continue trimming down or upslope to meet the tractor mowing limits as directed by the Project Manager.

  - The limits of trimming extend to the fence line in cases where a fence is located on the top of a slope at a bridge underpass.

- Vegetation Removal: Place trimmed vegetation in adjacent natural areas on the right of way within five (5) days of trimming. When no adjacent natural tree area exists, remove this vegetation from the right of way within five (5) days. Disposal shall be in accordance with applicable laws and regulations. Trimmed vegetation may be chipped and evenly distributed on non-mowable slopes on the right of way.

- Multiple Trunk Trimming: In areas where trimming occurs, trim all stems of multiple trunk vegetation that is 3 inches in caliper (dbh) or less.

- Planted and Established Ground cover: Consult with the Project Manager prior to trimming a non-mowable slope that is covered with a planted or established ground cover such as Lespedeza, Weeping Love Grass, or a native grass. If the Project Manager requires the groundcover to remain, trim all other vegetation in the groundcover.

4.04       MISCELLANEOUS MOWING COMPLETE REQUIREMENTS

    - Unless others directed in writing by the Project Manager, do not perform work in the following areas:

      - Construction project limits:  In the event rehabilitation or improvement project is under construction or will be under construction where mowing is scheduled, that portion of the Mowing Cycle will be deleted at the direction of the Project Manager. The section(s) of roads deleted from the list may be added back to the list at the first available Mowing Cycle following completion of said project.

2.  Wildflower plots:  Existing wildflower plots will be marked by the Project Manager and are not to be mowed until directed to do so, but will be trimmed as instructed, by the Project Manager.

    - Perform all scheduled mowing operations during the hours of daylight. No work will be allowed on holidays or weekends in which a holiday falls on a Monday or Friday.

    - Do not allow any debris to be thrown onto the road surface by the mowing equipment or by personnel.

    - Mowing equipment, while in operation, must be horizontal with the mowing surface. Tractor Mowers operating in a vertical position parallel to a traveled way is prohibited.

    - Equipment left on the right of way is the responsibility of the Contractor. Do not park equipment on the right of way/County property more than 5 working days in the same location.

      - Park no equipment in the median.

2. Park equipment behind guardrail or in other protected areas where such areas exist. Otherwise, park equipment a minimum of 30 feet from the edge of pavement.

    - Suspend operations if weather conditions are such that mowing operations cannot be carried out in an effective manner. If such suspension occurs, notify the Project Manager immediately. Likewise, the Project Manager may order the suspension of mowing operations whenever, in their judgment, present weather conditions are such that mowing operations cannot be carried out in a safe and effective manner.

    - In the event that any damage to roadside obstacles occur during and is caused by the Tractor Mowing or Trimming operations, repair or replace the damaged item with a like item at the Contractor's expense. In the event that damage occurs during and is caused by the Tractor Mowing or Trimming operations and said damage has to be repaired or replaced utilizing Department property or personnel, the cost of the repair or replacement shall be calculated and deducted from the Contractor's payment.

    - The removal of cut material, such as the baling of hay, is prohibited.

I.  Herbicide use is strictly prohibited and may result in the termination of contract.

  - TRAFFIC CONTROL REQUIREMENTS

    - At a minimum, abide by the Manual of Uniform Traffic Control Devices (MUTCD), current edition, for traffic control guidance.

    - Abide by the Mowing Work Zone Details (See Attachments “D”), for continuous mowing operations.

      - Perform all Tractor Mowing as a continuous mowing operation as described in these Specifications and as shown on attached "Continuous Mowing" details. Mow only in the direction of traffic.

- Any signs used must be of rigid, one piece construction covered with suitable sheeting and legend (engineering grade). Roll-up type signs may be used if they meet MUTCD Standard.

- Do not cross the median or other divided highways. Equip (1) support vehicles, such as pickups and service trucks with a minimum of one revolving or strobe light, with amber lens.

- Use of a shadow vehicle to buffer the Tractor Mowing operation when it is on or within three feet of the travel way. (See Attachment E- Mowing Work Zone Details)

- Equip support vehicles used as shadow vehicle with a 48 inch square "Watch for Mowers" sign with an 18 inch slow moving emblem attached and mounted on the rear of the Shadow vehicle.

- Equip a shadow vehicle with a minimum of one (1) revolving or strobe type light, with amber lens.

    - Do not exceed a ten mile work zone. Only one Tractor Mowing operation in a work zone will be allowed. Additional work zones will be allowed, separated by a minimum of two (2) miles. Do not exceed 20 miles of mowing without completing trimming.

      - Signs- Place a "Watch for Mowers" sign with flags attached at each end of the work zone in the direction of traffic on both the median and outside shoulders.

    - Buffer tractor mowers traveling within three (3) feet of the traveled way, crossing lanes, traveling paved shoulders, moving equipment to other locations (dead heading), by a Shadow Vehicle.

      - Operations may be restricted when, in the opinion of the Project Manager, the continuance of the work would seriously hinder traffic or is unsafe.

2.   Move equipment or materials on or across the traveled way at other times in a manner as not to unduly interfere with traffic. There shall be reduction in the total number of available traveled ways. Schedule and arrange the Work to ensure the least inconvenience and the utmost in safety to the traveling public and to the Contractor's and the Department's forces.

    - Notify the Project Manager, in writing, of any accident that involves the Contractor while fulfilling this Contract.

      - Submit an incident report to the Project Manager within forty eight (48) working hours after incident which described the accident in full, names of those involved with contact phone numbers, and extent of injury and damage.

2. Employees shall carry business cards that have the name, address and phone numbers of the Contractor upon request of individuals involved in an accident.

4.06    QUALITY ACCEPTANCE

    - The height of all vegetation after any Tractor Mowing or Trimming operations shall be a 6- inch mowing height, except for woody vegetation between 2 1/2 to 3 inches in caliper (dbh), which shall be trimmed to less than 6 inches. Heights will be determined  by taking the average  of three  (3) measurements  at the following  locations:

- One fourth of the distance from the edge of the pavement (edge of payment includes paved shoulders) to the shoulder point.

- One half of the distance from the shoulder point to the bottom of the ditch or toe of the slope, whichever is applicable; and

- At the midpoint of the farthest mowing pass from the edge of the pavement. The Department will give an allowance of two (2) inches higher or lower than the specified height of six (6) inches when averaging the measurements.

- Progress and performance will be monitored by the Project Manager, using the approved plan from the Contractor as a guide to verity the Centerline Miles mowed, and the fifty percent (50%) Intermediate Completion Dates. The Contractor will maintain a log of Mowing Complete and will verity the completion of the work with the Project Manager upon Mowing Complete.

- All routes where mowing/trimming have been completed must be approved/accepted by the Project Manager prior to any payment. At no time will payment be made for any work not yet performed.

  - Mowing Complete will be left uniform in appearance after the mowers have passed.

  - Any areas that are not mowed due to insufficient overlap of the mowers or depression of the vegetation by the mowing equipment will not be accepted and must be mowed again.

  - Performance deficiencies will be reported to the Contractor by facsimile within seventy-two (72) hours of identification.

  - Correct Mowing/trimming Quality Acceptance deficiencies no later than forty-eight (48) hours following receipt of said notice.

| ATTACHMENT “B” |  |  |  |
| --- | --- | --- | --- |
|  |  |  | Length |
| Street | From | To | CLM |
| Alexander Parkway | Ga Hwy 5 | Cougar Trail | 1.11 |
| Andy Mountain Road | Conners Road | Nalley Road | 0.69 |
| Anneewakee Road | Chapel Hill Road | Ga Hwy 92 | 4.01 |
| Banks Mill Road | Post Road | Ga Hwy 5 | 3.64 |
| Bearden Road | Gresham Road | Maroney Mill Road | 0.96 |
| Beechwood Drive | Mount Vernon Road | Lee Road | 0.81 |
| Ben Hill Road | Hwy 78 | North Sweetwater Road | 2.64 |
| Berea Road | Ga Hwy 5 | Daniel Mill Road | 3.67 |
| Big A Road | Ga Hwy 5 | Ga Hwy 166 | 3.81 |
| Blairs Bridge Road | Thornton Road | Mount Vernon Road | 1.08 |
| Bomar Road | Chapel Hill Road | Ga Hwy 92 | 3.65 |
| Boyd Road | Ben Hill Road | North Burnt Hickory Road | 0.99 |
| Brewer Road | Mann Road | City Limits (Villa Rica) | 2.13 |
| Bright Star Road | City Limits | Ga Hwy 5 | 3.05 |
| Brittain Road | Cedar Mountain Road | High Point Road | 1.52 |
| Camp Road | Ga Hwy 166 | Post Road | 0.77 |
| Cantrell Road | Phillips Mill Road | Five Notch Road | 1.62 |
| Capps Ferry Road | Ga Hwy 166 | The County Line | 3.15 |
| Cedar Mountain Road | Mann Road | Dead End | 4.54 |
| Cedar Terrace Road | Mount Vernon Road | Lee Road | 0.79 |
| Central Church Road | Bright Star Road | Chapel Hill Road | 3.94 |
| Chicago Avenue | Strickland Rd | Cedar Mountain Rd | 0.81 |
| Chapel Hill Road | Interstate I 20 | Ga Hwy 166 | 6.54 |
| Cole Road | Liberty Road | Ephesus Church Road | 1.22 |
| Conners Road | Post Road | The City Limits (Villa Rica) | 3.03 |
| Cool Springs Road | Post Road | Dead End | 1.12 |
| Cougar Trail | Alexander Parkway | Mason Creek Road | 0.43 |
| Cowan Mill Road | Mason Creek Road | East Baggett Road | 2.58 |
| Daniel Mill Road | Post Road | Banks Mill Road | 2.58 |
| Dorris Road | Sweetwater Church Road | Cedar Mountain Road | 1.67 |
| Dorsett Shoals Road | Ga Hwy 5 | Chapel Hill Road | 6.36 |
| Douglas Hill Road | Thornton Road | Factory Shoals Road | 1.94 |
| East Baggett Road | Cowan Mill Road | Dead End | 0.77 |
| East Carroll Road | Tyree Road | Carroll County Line | 1.73 |
|  | ATTACHMENT “B” |  |  |
| FY 2019 Mowing and Shoulder Maintenance Road List |  |  |  |
|  |  |  | Length |
| Street | From | To | CLM |
| East County Line Road | North/South County Line Road | Lee Road | 1.30 |
| East Union Hill Road | Daniel Mill Road | Post Road | 0.92 |
| Ephesus Church Road | Post Road | The County Line | 4.75 |
| Factory Shoals Road | Thornton Road | Mount Vernon Road | 3.02 |
| Five Notch Road | Cantrell Road | Dead End | 1.25 |
| Flint Hill Road | Post Road | Ga Hwy 5 | 1.53 |
| Florence Drive | Hwy 78 | Jr. High Drive | 0.50 |
| Fouts Mill Road | Ga Hwy 166 | Punkintown Road | 1.00 |
| Gresham Road | North Burnt Hickory Road | The County Line | 0.66 |
| Hannah Road | Ga Hwy 166 | East Carroll Road | 2.16 |
| High Point Road | Brittain Road | Ga Hwy 61 | 3.60 |
| Huey Road | Maroney Mill Road | City Limits | 1.38 |
| John West Road | Bright Star Road | Hwy 78 | 1.64 |
| Johnston Road | Mason Creek Road | Pool Road | 1.80 |
| Jr. High Drive | Florence Drive | South Sweetwater Road | 0.34 |
| Kilroy Lane | Ga Hwy 5 | Big A Road | 1.65 |
| Kings Highway | Ga Hwy 5 | Big A Road | 4.23 |
| Lake Greystone Road | Old Douglasville Road | Dead End | 0.92 |
| Lake Monroe Road | Pope Road | Ga Hwy 92 | 2.04 |
| Lee Road | Ga Hwy 92 | South Sweetwater Road | 3.47 |
| Liberty Road | Poole Road | Post Road | 6.97 |
| Lithia Way | Florence Drive | North County Line Road | 0.63 |
| Mann Road | Hwy 78 | Brewer Road | 4.86 |
| Maroney Mill Road | Hwy 78 | Ga Hwy 92 | 2.96 |
| Mason Creek Road | Post Road | Dorsett Shoals | 5.09 |
| Mattie McCoy Lane | Conners Road | Richardson Road | 0.95 |
| Maxham Road | Thornton Road | Old Alabama Road | 1.43 |
| Mckown Mill Rd | South Burnt Hickory Rd | Riley Rd | 1.98 |
| Midway Road | Hwy 78 | Ga Hwy 92 | 2.19 |
| Miller Street | Hwy 78 | South Sweetwater Road | 1.56 |
| Monier Avenue | Mount Vernon Road | Lee Road | 0.92 |
| Mount Vernon Road | Ga Hwy 92 | Thornton Road | 5.57 |
| Nalley Road | Timmons Circle | Andy Mountain Road | 0.59 |
| North Baggett Road | Hwy 78 | Dead End | 0.98 |
| North Burnt Hickory | Hwy 78 | Cobb County Line | 2.40 |
|  | ATTACHMENT “B” |  |  |

| FY 2019 Mowing and Shoulder Maintenance Road List |  |  |  |
| --- | --- | --- | --- |
|  |  |  | Length |
| Street | From | To | CLM |

| North County Line Road | East County Line Road | Hwy 78 | 3.05 |
| --- | --- | --- | --- |
| North Flat Rock Road | Cedar Mountain Road | Walton Store Road | 0.97 |
| North Sweetwater Road | Hwy 78 | The County Line | 1.05 |
| Old Alabama Road | Maxham Road | Cobb County Line | 1.02 |
| Old Beaulah Road | Riley Road | Hwy 78 | 1.27 |
| Old Douglasville Road | North Sweetwater Road | Ben Hill Road | 2.01 |
| Old Lee Road | Lee Road | Ga Hwy 92 | 0.66 |
| Old Lower River Road | Hwy 166 | Riverside Parkway | 0.83 |
| Phillips Mill Road | Capps Ferry Road | Dead End | 1.89 |
| Pool Road | Ephesus Church Road | Ga Hwy 5 | 4.29 |
| Poole Mill Road | Pool Road | Banks Mill Road | 1.16 |
| Poole Road | Liberty Road | West Tyson Road | 0.62 |
| Pope Road | Ga Hwy 92 | Anneewakee Road | 3.34 |
| Prestley Mill Rd | Campbelton Street | Slater Mill Rd | 1.83 |
| Post Road | Hwy 78 | The County Line | 11.68 |
| Punkintown Road | Kings Hwy | Fouts Mill Road | 1.55 |
| Reid Road | Central Church Road | Yeager Road | 0.61 |
| Reynolds Road | Stewarts Mill Road | Central Church Road | 1.41 |
| Richardson Road | Conners Road | Mattie McCoy Lane | 2.40 |
| Riley Road | Hwy 78 | North County Line Road | 2.05 |
| Riverside Parkway | Hwy92 | Thornton Rd | 6.23 |
| Shell Road | Ga Hwy 166 | Tyree Road | 1.37 |
| Simon Road | Bomar Road | Anneewakee Road | 1.36 |
| Sinyard Road | Beechwood Drive | Skyview Drive | 0.62 |
| Skyview Drive | Thornton Road | South Sweetwater Road | 2.79 |
| Smith Road | Conners Rd | Richardson Road | 1.00 |
| South Baggett Road | Hwy 78 | Mason Creek Road | 1.62 |
| South Burnt Hickory | Hwy 78 | I-20 | 1.52 |
| South County Line Road | East County Line Road | Lee Road | 0.90 |
| South Giles Road | Ga Hwy 5 | Banks Mill Road | 0.88 |
| South River Road | Hwy 166 | Hwy 166 | 2.15 |
|  | ATTACHMENT “B” |  |  |

| FY 2019 Mowing and Shoulder Maintenance Road List |  |  |  |
| --- | --- | --- | --- |
|  |  |  | Length |
| Street | From | To | CLM |

| South Sweetwater Road | Hwy 78 | Lee Road | 2.23 |
| --- | --- | --- | --- |
| Stewart Mill Road | Central Church Road | Chapel Hill Road | 2.39 |
| Temple Street | North Sweetwater Road | Cobb County Line | 1.03 |
| Timmons Circle | Tyson Road | Nalley Road | 1.22 |
| Tyree Road | Liberty Road | Ga Hwy 166 | 4.80 |
| Union Grove Road | North Sweetwater Road | Ben Hill Road | 0.95 |
| West Banks Mill Road | Liberty Road | Post Road | 0.75 |
| West Chapel Hill Road | Central Church Road | Ga Hwy 166 | 2.85 |
| West County Line Road | Midway Road | North/South County Line Road | 1.60 |
| West Stewart Mill Road | Ga Hwy 5 | Stewarts Mill Road | 1.14 |
| West Union Hill Road | North Helton Road | Ephesus Church Road | 1.63 |
| West Yeager Road | Yeager Road | Fouts Mill Road | 0.90 |
| Yancey Road | Stewarts Mill Road | Dorsett Shoals | 2.48 |
| Yeager Road | Kings Hwy | West Chapel Hill Road | 4.80 |

ATTACHMENT"C"

EVALUTION CRITERIA FOR TRACTOR MOWING

AND TRIMMING CONTRACTS

In addition to your monetary offer, the following will be used as Evaluation Criteria of your bid. Read the Evaluation Criteria carefully and submit all requested information. This information will be required prior to any evaluations being made. Failure to comply will result in bid rejection.

- Submit with the bid a written certification that you will possess all necessary equipment and personnel to fulfill the terms of these Specifications and be ready to begin upon award of a Contract. The Department will make every effort to award the contract two (2) weeks prior to beginning date of the 1st mowing cycle.

  - Submit with the bid a minimum of three (3) references from present or past successful commercial contract customers, one of which should be from a contract of similar size and scope as contemplated by this Contract. Include a description of the mowing duties performed, the size of the mowing services provided described in terms of acreage. Provide a contact name and phone number for all references provided.

  - Submit with the bid a list of all company owned or leased equipment intended for use. A current lease agreement, with equipment list, contingent upon contract award will satisfy this requirement.

  - Contractors identified and documented as having previously been in significant non- compliance or default of State of Georgia Contracts or other contract specification requirements shall not be considered for award.

  - To be considered for award, Contractors shall not have been found to be in non-compliance with state or federal laws and /or regulations relevant to the payment of taxes, employee salaries, or workers compensation.

  - Submit with the bid, evidence of being commercial mowing contractor for a minimum of three (3) years. Copies of current and previous year's business licenses will satisfy this requirement.